Arc Circle Fill

Installation & Setup Guide

For Autodesk Fusion 360  |  v1.2.4

# Overview

Arc Circle Fill is a Fusion 360 add-in that places equally-spaced tangent circles along sketch arcs, calculates coordinates based on insert diameter and cusp height, and sends the results directly to your mobile device for use at the machine.

# What You Need

Before installing, make sure you have the following:

- Autodesk Fusion 360 — already installed

- Git for Windows — free download (see Section 2)

- A GitHub account — free (see Section 3)

- Google Chrome — on your mobile device for the Coord Stepper app

You do not need to install Python, Node.js, or any other software. Git is the only additional requirement.

# 1.  Installing the Fusion 360 Add-In

## Step 1 — Copy the add-in files

Download the two add-in files:

- ArcCircleAddin.py

- ArcCircleAddin.manifest

Copy both files into this folder on your PC (create it if it does not exist):

C:\Users\<YourUsername>\AppData\Roaming\Autodesk\Autodesk Fusion 360\API\AddIns\ArcCircleAddin\

Replace <YourUsername> with your Windows login name. Both files must be in a folder named ArcCircleAddin.

## Step 2 — Load the add-in in Fusion 360

- Open Fusion 360

- Go to Utilities → Scripts and Add-Ins  (or press Shift + S)

- Click the Add-Ins tab

- Find ArcCircleAddin in the list and click Run

- Tick Run on Startup so it loads automatically next time

The button "Arc Circle Fill" will now appear in the Sketch → Modify panel. It is only visible when you are inside an open sketch.

If the button does not appear in Sketch → Modify, look in the Utilities toolbar — it may appear there depending on your Fusion version.

# 2.  Installing Git for Windows

Git is required to push coordinates to GitHub so they can be viewed on your mobile device.

- Go to: https://git-scm.com/download/win

- Download the 64-bit installer

- Run the installer — all default options are fine, just click Next through each screen

- Once installed, open Command Prompt and type:

git --version

You should see something like:  git version 2.xx.x.windows.1

If you see "git is not recognized", restart your computer and try again.

# 3.  GitHub Account Setup

GitHub is where the coordinates are stored so your phone can access them. You only need to do this once.

## Create an account

- Go to https://github.com and click Sign up

- Choose a username, enter your email and password

- Verify your email address

## Authenticate Git on your PC

The first time you push coordinates to GitHub, a browser window will open asking you to sign in. This only happens once — Git remembers your credentials afterwards.

If you are installing this on a workmate's PC, they will need to sign in to their GitHub account the first time they push. All coordinates will be sent to the shared GitHub repository.

# 4.  Setting Up the Mobile Coord Stepper

The Coord Stepper is a web app that runs in your phone's browser — no app store needed.

## Open the app on your phone

- On your phone, open Google Chrome

- Go to: https://mattdennison77.github.io/arc-circle-fill/stepper.html

- Bookmark this page for quick access

## Add to Home Screen (optional but recommended)

- In Chrome, tap the three dots menu (top right)

- Tap Add to Home Screen

- Tap Add — the app icon will appear on your home screen

The app will automatically fetch the latest coordinates from GitHub each time it opens. Tap the Refresh button after running the add-in to load new coordinates.

# 5.  Using Arc Circle Fill

## Running the add-in

- Open a sketch in Fusion 360 that contains at least one arc

- Double-click the sketch to enter edit mode

- Click Arc Circle Fill from the Sketch → Modify panel

- Enter the insert diameter (mm)

- Choose spacing method: Yes for cusp height, No for manual spacing

- If XZ plane is detected: choose Yes or No for Lathe Mode

- Choose Push to mobile? — Yes to send coordinates to your phone

- Choose Export PDF? — Yes to save and open a PDF report

## On your phone

- Open the Coord Stepper app (or refresh the page)

- Tap ↻ Refresh to load the latest coordinates

- Use ▲ Prev and ▼ Next buttons (or swipe) to step through positions

- Pinch to zoom and drag on the arc preview

# 6.  Troubleshooting

## Add-in button does not appear

- Make sure you are inside an open sketch (double-click the sketch first)

- Check the add-in is loaded: Utilities → Scripts and Add-Ins → Add-Ins tab

- Try reloading: click Stop then Run in the Add-Ins dialog

## GitHub push failed

- Check Git is installed: open Command Prompt and type git --version

- Make sure you are connected to the internet

- On first use, a browser window will open — sign in to GitHub to authorise

## Phone shows old coordinates after refresh

- Wait 2-3 minutes after the add-in pushes — GitHub can take a moment to update

- Try closing and reopening Chrome on your phone

- Clear Chrome cache: Settings → Privacy → Clear browsing data → Cached images

## PDF does not open automatically

- Check the file was saved to: Documents\ArcCircleFill\arc_circle_report.pdf

- Open it manually from that folder

Arc Circle Fill  |  For support contact your system administrator  |  v1.2.4